0 00h Bank Select (Controller # 32 more commonly used) 
1 01h Modulation Wheel 
2 02h Breath Contoller 
3 03h Undefined 
4 04h Foot Controller 
5 05h Portamento Time 
6 06h Data Entry MSB 
7 07h Main Volume 
8 08h Balance 
9 09h Undefined 
10 0Ah Pan 
11 0Bh 0Ch 
12 0Ch Effect Control 1 
13 0Dh Effect Control 2 
14-15 0E-0Fh Undefined 
16-19 10-13h General Purpose Controllers (Nos. 1-4) 
20-31 14-1Fh Undefined 
32-63 20-3Fh LSB for Controllers 0-31 (rarely implemented) 
64 40h Damper Pedal (Sustain) [Data Byte of 0-63=0ff, 64-127=On] 
65 41h Portamento 
66 42h Sostenuto 
67 43h Soft Pedal 
68 44h Legato Footswitch 
69 45h Hold 2 
70 46h Sound Controller 1 (default: Sound Variation) 
71 47h Sound Controller 2 (default: Timbre/Harmonic Content) 
72 48h Sound Controller 3 (default: Release Time) 
73 49h Sound Controller 4 (default: Attack Time) 
74 4Ah Sound Controller 5 (default: Brightness) 
75-79 4B-4Fh Sound Controller 6-10 (no defaults) 
80-83 50-53h General Purpose Controllers (Nos. 5-8) 
84 54h Portamento Control 
85-90 55-5Ah Undefined 
91 5Bh Effects 1 Depth (previously External Effects Depth) 
92 5Ch Effects 2 Depth (previously Tremolo Depth) 
93 5Dh Effects 3 Depth (previously Chorus Depth) 
94 5Eh Effects 4 Depth (previously Detune Depth) 
95 5Fh Effects 5 Depth (previously Phaser Depth) 
96 60h Data Increment 
97 61h Data Decrement 
98 62h Non-Registered Parameter Number LSB 
99 63h Non-Registered Parameter Number LSB 
100 64h Registered Parameter Number LSB 
101 65h Registered Parameter Number MSB 
102-120 66-78h Undefined 

So it looks like you were right about portamento being 65!